Itop-4412 的i2c总线驱动目录

-> Device Drivers

-> I2C support (I2C [=y])

-> I2C Hardware Bus support

< > S3C2410 I2C Driver

设备(适配器)：dev-i2c*.c	驱动(适配器)：i2c-s3c2410.c

设备注册：mach-itop4412.c			2781行

驱动注册：static struct  platform_device_id  s3c24xx_driver_ids[] = {

{

.name		= "s3c2410-i2c",

.driver_data	= TYPE_S3C2410,

}, {

.name		= "s3c2440-i2c",

.driver_data	= TYPE_S3C2440,

}, {

.name		= "s3c2440-hdmiphy-i2c",

.driver_data	= TYPE_S3C2440_HDMIPHY,

}, { },

};

static struct  platform_driver  s3c24xx_i2c_driver = {

.probe		= s3c24xx_i2c_probe,

.remove		= s3c24xx_i2c_remove,

.id_table	= s3c24xx_driver_ids,

.driver		= {

.owner	= THIS_MODULE,

.name	= "s3c-i2c",

.pm	= S3C24XX_DEV_PM_OPS,

},

};

根据platform_device_id中的name来匹配i2c设备。每发现一个匹配的适配器，就调用一次

s3c24xx_i2c_probe函数。

Itop-4412 的电容触摸屏驱动目录

-> Device Drivers

-> Input device support

-> Touchscreens (INPUT_TOUCHSCREEN [=y])

<>FT5X0X based touchscreens

设备(I2c设备)：mach-itop4412.c		驱动(I2c设备)：ft5x06.c

I2c设备注册：static struct  i2c_board_info  i2c_devs3[]  __initdata = {

/* add by cym 20130318 support for FT5X0X TouchScreen */

#if defined(CONFIG_TOUCHSCREEN_FT5X0X)

{

I2C_BOARD_INFO("ft5x0x_ts", 0x70>>1),

.irq = IRQ_EINT(4),

.platform_data = &ft5x0x_pdata,

},

#endif

/* end add */

};

i2c_register_board_info(3, i2c_devs3, ARRAY_SIZE(i2c_devs3));

在第三个适配器里添加该I2c设备。

I2c驱动注册：

static const struct  i2c_device_id  ft5x0x_ts_id[] = {

{ "ft5x0x_ts", 0 },

{ }

};

static struct  i2c_driver  ft5x0x_ts_driver = {

.probe		= ft5x0x_ts_probe,

.remove		= __devexit_p(ft5x0x_ts_remove),

.id_table	= ft5x0x_ts_id,

.driver	= {

.name	= FT5X0X_NAME,

.owner	= THIS_MODULE,

},

};

i2c_add_driver(&ft5x0x_ts_driver);

找到名为"ft5x0x_ts"的i2c设备后调用ft5x0x_ts_probe函数。

本驱动程序由于讯为自带tslib版本问题，只能以单点触控方式上报。

在测试ts_bus的时候先把mach-itop4412中的i2c3注释掉

1.//&s3c_device_i2c3,

2.//s3c_i2c3_set_platdata(NULL);

3.//i2c_register_board_info(3,i2c_devs3,ARRAY_SIZE(i2c_devs3));

再用之前写的触摸屏的驱动程序测试。由于ft5406的i2c存储很多都是只读的，所以选取的时候可以选后面的那些地址测试，如：85h;

ft5406的存储地址表如下：